| Python Code | Annotations |
| --- | --- |
| import pandas as pd import numpy as np import matplotlib.pyplot as plt import seaborn as sns sns.set_style('whitegrid') sns.set_palette('deep') | This code imports the required Python libraries so they can be used in the program. Pandas is used for data handling, NumPy is used for numerical operations, matplotlib is used for plotting graphs, and seaborn is used to improve the visual appearance of the plots. These lines also set the style of the plots to make them more readable and visually appealing. |
| # Load the datasets nhts_data = pd.read_csv('NHTS.csv') ngsim_data = pd.read_csv('NGSIM.csv') # Look at the first few rows print(nhts_data.head()) print(ngsim_data.head()) | These lines load the NHTS and NGSIM datasets into Python using pandas and store them as dataframes for further analysis. The head() function is then used to display the first few rows of each dataset, allowing for a quick preview and verification of the data. |
| # column names print("NHTS columns:") print(nhts_data.columns) print("NGSIM columns:") print(ngsim_data.columns) | These lines print the column names of the NHTS and NGSIM datasets. This helps identify the available variables and ensures the correct columns are selected for analysis. |
| # Keep only the variables named in the methods section # NHTS variables: travel_day, vehicle_age, vehicle_type nhts_subset = nhts_data[['travel_day', 'vehicle_age', 'vehicle_type']].dropna() # NGSIM variables: Time, trajectory_number, leader/follower speed, acceleration, and position ngsim_subset = ngsim_data[['trajectory_number', 'Time', 'leader_speed(m/s)', 'follower_speed(m/s)', 'leader_acc(m/s^2)', 'follower_acc(m/s^2)', 'leader_position(m)', 'follower_position(m)']].dropna() print(nhts_subset.head()) print(ngsim_subset.head()) | This section selects only the relevant variables from the NHTS and NGSIM datasets based on the methods section. It also removes any missing values using dropna() to ensure clean and consistent data for analysis. The resulting subsets are then displayed to verify the selected variables. |
| # Bar Chart Count trips by travel day travel_day_counts = nhts_subset['travel_day'].value_counts().sort_index() plt.figure(figsize=(10,7)) travel_day_counts.plot(kind='bar', color='yellow', edgecolor='black') plt.xlabel('Travel Day', fontsize=12) plt.ylabel('Number of Trips', fontsize=12) plt.title('Number of Trips by Travel Day', fontsize=14) plt.xticks(rotation=50) plt.show() | This section creates a bar chart that shows the number of trips for each day of the week. The value_counts() function counts how many times each category appears, and sort_index() organizes the days in order. The plot displays travel_day on the x-axis and the number of trips on the y-axis. |
| #histogram Vehicle age distribution plt.figure(figsize=(10,7)) plt.hist(nhts_subset['vehicle_age'], bins=24, color='green', edgecolor='black', alpha=0.9) plt.xlabel('Vehicle Age (years)', fontsize=12) plt.ylabel('Count', fontsize=12) plt.title('Distribution of Vehicle Age', fontsize=14) plt.show() | This section creates a histogram to show the distribution of vehicle ages. The histogram groups vehicle ages into bins and displays how frequently each range appears. This helps identify patterns such as whether most vehicles are newer or older. |
| # Boxplot Vehicle age by vehicle type plt.figure(figsize=(12,7)) nhts_subset.boxplot(column='vehicle_age', by='vehicle_type') plt.xlabel('Vehicle Model', fontsize=12) plt.ylabel('Vehicle Age (years)', fontsize=12) plt.title('Vehicle Age by Vehicle Model', fontsize=14) plt.suptitle(' ') plt.xticks(rotation=50) plt.show() | This section creates a boxplot that compares vehicle age across different vehicle types. Each box represents a category of vehicle_type and shows the median, spread, and outliers. This helps compare how vehicle age differs between categories. |
| # one trajectory for the time-series plots trajectory_number = 3 data_subset = ngsim_subset[ngsim_subset['trajectory_number'] == trajectory_number].reset_index(drop=True) print(data_subset.head()) | This section filters the NGSIM dataset to only include one trajectory number. This simplifies the analysis and makes the time-series plots easier to interpret by focusing on a single vehicle interaction. |
| # Create spacing variable data_subset['spacing'] = data_subset['leader_position(m)'] - data_subset['follower_position(m)'] print(data_subset[['Time', 'leader_position(m)', 'follower_position(m)', 'spacing']].head()) | This line creates a new variable called spacing by subtracting the follower’s position from the leader’s position. This represents the distance (gap) between the two vehicles and is used to analyze vehicle-following behavior and spacing over time. |
| # 1st Time series plot 1 Leader speed and follower speed vs time plt.figure(figsize=(12,5)) sns.lineplot(data=data_subset, x='Time', y='leader_speed(m/s)', label='Leader Speed', color='pink') sns.lineplot(data=data_subset, x='Time', y='follower_speed(m/s)', label='Follower Speed', color='purple') plt.xlabel('Time (s)', fontsize=12) plt.ylabel('Speed (m/s)', fontsize=12) plt.title('Leader and Follower Speed vs Time', fontsize=14) plt.legend() plt.show() | This section creates a time-series plot comparing leader speed and follower speed over time. Two lines are plotted on the same graph, allowing direct comparison of how the follower reacts to the leader’s movement. |
| # 2nd Time series plot Leader acceleration and follower acceleration vs time plt.figure(figsize=(12,5)) sns.lineplot(data=data_subset, x='Time', y='leader_acc(m/s^2)', label='Leader Acceleration', color='green') sns.lineplot(data=data_subset, x='Time', y='follower_acc(m/s^2)', label='Follower Acceleration', color='blue') plt.xlabel('Time (s)', fontsize=12) plt.ylabel('Acceleration (m/s^2)', fontsize=12) plt.title('Leader and Follower Acceleration vs Time', fontsize=14) plt.legend() plt.show() | This section creates a time-series plot comparing leader acceleration and follower acceleration. This helps show how both vehicles respond to driving conditions and how their acceleration patterns differ over time. |
| # EXTRA 3rd Time series plot Spacing vs time plt.figure(figsize=(12,5)) plt.plot(data_subset['Time'], data_subset['spacing'], color='orange', linewidth=1.5) plt.xlabel('Time (s)', fontsize=12) plt.ylabel('Spacing (m)', fontsize=12) plt.title('Spacing Between Leader and Follower vs Time', fontsize=14) plt.show() | This section creates a plot showing how spacing between the vehicles changes over time. This helps analyze following behavior and whether the follower maintains a safe distance |
| # Defining IDM acceleration function def idm_acceleration(cs, gap, delta_v, d_speed, mini_g, t, max_a, b, delta): d_gap = mini_g + cs * t + (cs * delta_v) / (2 * np.sqrt(max_a * b)) d_gap = max(d_gap, mini_g) output = max_a * (1 - (cs / d_speed) ** delta - (d_gap / gap) ** 2) return output | This section defines the IDM (Intelligent Driver Model) acceleration function. The function calculates acceleration based on speed, spacing, and relative velocity between vehicles. It is used to simulate how a driver would respond to traffic conditions. |
| def idm_acceleration(v, s, delta_v, v0, s0, T, a, b, delta): s_star = s0 + v * T + (v * delta_v) / (2 * np.sqrt(a * b)) s_star = max(s_star, s0) acceleration = a * (1 - (v / v0) ** delta - (s_star / s) ** 2) return acceleration # Data for IDM simulation time_data = data_subset['Time'].values leader_position = data_subset['leader_position(m)'].values leader_speed = data_subset['leader_speed(m/s)'].values actual_follower_position = data_subset['follower_position(m)'].values actual_follower_speed = data_subset['follower_speed(m/s)'].values actual_follower_acc = data_subset['follower_acc(m/s^2)'].values dt = 0.1 # IDM parameters d_speed = 30 mini_g = 2 t = 1.5 max_a = 1.0 b = 1.5 delta = 4 # Initialize arrays n_steps = len(time_data) sim_position = np.zeros(n_steps) sim_speed = np.zeros(n_steps) sim_acc = np.zeros(n_steps) # Start the simulation using the actual follower starting values sim_position[0] = actual_follower_position[0] sim_speed[0] = actual_follower_speed[0] | This code defines the IDM acceleration model, loads the required vehicle data into arrays, sets the simulation parameters, and initializes the variables needed to run the vehicle-following simulation. |
| # Run the IDM simulation for i in range(n_steps - 1): gap = max(leader_position[i] - sim_position[i], 0.1) delta_v = sim_speed[i] - leader_speed[i] sim_acc[i] = idm_acceleration(sim_speed[i], gap, delta_v, d_speed, mini_g, t, max_a, b, delta) sim_speed[i + 1] = max(sim_speed[i] + sim_acc[i] * dt, 0) sim_position[i + 1] = sim_position[i] + sim_speed[i] * dt print("Simulation complete!") | This loop runs the IDM simulation over time. At each step, it calculates the gap between the leader and simulated follower, computes the relative speed, and uses the IDM function to determine acceleration. The follower’s speed and position are then updated for the next time step. This process models how the follower vehicle responds to the leader’s movement over time. |
| # Compare actual and simulated follower position plt.figure(figsize=(12,7)) plt.plot(time_data, leader_position, label='Leader Position', color='brown') plt.plot(time_data, actual_follower_position, label='Actual Follower Position', color='green') plt.plot(time_data, sim_position, label='Simulated Follower Position', color='yellow') plt.xlabel('Time (s)', fontsize=12) plt.ylabel('Position (m)', fontsize=12) plt.title('Actual and Simulated Vehicle Position vs Time', fontsize=14) plt.legend() plt.grid(True, alpha=0.6) plt.show() | This loop runs the IDM simulation over time. At each time step, the code calculates the gap between the leader and follower vehicles, computes the follower’s acceleration using the IDM equation, and updates the follower’s speed and position. This process models how the follower vehicle responds dynamically to the leader over time. |
| # Compare actual and simulated follower speed plt.figure(figsize=(12,7)) plt.plot(time_data, actual_follower_speed, label='Actual Follower Speed', color='purple') plt.plot(time_data, sim_speed, label='Simulated Follower Speed', color='green') plt.xlabel('Time (s)', fontsize=12) plt.ylabel('Speed (m/s)', fontsize=12) plt.title('Actual and Simulated Follower Speed vs Time', fontsize=14) plt.legend() plt.grid(True, alpha=0.6) plt.show() | This plot compares the actual and simulated follower vehicle speed over time. It helps evaluate how accurately the model predicts real driving behavior by showing differences between observed and simulated speed patterns. |
| # Compare actual and simulated follower acceleration plt.figure(figsize=(12,7)) plt.plot(time_data, actual_follower_acc, label='Actual Follower Acceleration', color='green') plt.plot(time_data, sim_acc, label='Simulated Follower Acceleration', color='red') plt.xlabel('Time (s)', fontsize=12) plt.ylabel('Acceleration (m/s^2)', fontsize=12) plt.title('Actual and Simulated Follower Acceleration vs Time', fontsize=14) plt.legend() plt.grid(True, alpha=0.6) plt.show() | This plot compares the actual and simulated follower vehicle acceleration over time. It helps evaluate how well the model captures real driving behavior by highlighting differences between observed and predicted acceleration patterns. |
| # Compare actual and simulated gap distance actual_gap = leader_position - actual_follower_position sim_gap = leader_position - sim_position plt.figure(figsize=(12,7)) plt.plot(time_data, actual_gap, label='actual Gap', color='green') plt.plot(time_data, sim_gap, label='Simulated Gap', color='red') plt.xlabel('Time (s)', fontsize=12) plt.ylabel('Gap Distance (m)', fontsize=12) plt.title('Actual and Simulated Gap Distance vs Time', fontsize=14) plt.legend() plt.grid(True, alpha=0.6) plt.show() | This plot compares the actual and simulated gap distance between the leader and follower vehicles over time. It helps evaluate how well the simulation model represents real vehicle-following behavior by highlighting differences between observed and predicted spacing. |